- Place the lid on the chamber and press the interlock button

- Open new crylas software ("C:\Program Files (x86)\CB_Control_HP\CB_Control_HP.exe")

- Press “connect” at the top

- Turn “Closed Loop Control” on

- Set trigger rate to 100Hz

- Turn laser “On’

- Enable Photomanupulation in Calibration tab

- Turn on shutter

- Turn on Crylas 532nm laser

- Set laser to 20%

- Set camera to area mode

- Set exposure to 500ms

- Select BP580-627 filter

\

- Go live

- Select right camera and begin calibration